Table 1:

_______________________________________________________________________________

|       Response File Format                                                  |

|_____________________________________________________________________________|

|Line #| Position | Field | Format   | Description                            |

|1-L   |   1-80   | -     |  a80     | General comments preceded by a #       |

|L+1   |   1      | 1     |  a1      | #                                      |

|      |   3-80   | 2     |  a78     | instrument type/description (KS36000,  |

|      |          |       |          | GS-13, etc.)                           |

|       Instrument Response Group Using Poles and Zeros (paz)                 |

|_____________________________________________________________________________|

|L+2-K |   1-80   | 1     |  a80     | comments (preceded by a "#")           |

|K+1   |   1-12   | 1     |  a12     | response source                        |

|      |          |       |          | (theoretical or measured)              |

|      |  14-15   | 2     |  i2      |sequence number                         |

|      |  17-28   | 2     |  a12     |description (instrument,                |

|      |          |       |          |anti-alias, etc.)                       |

|      |  30-35   | 3     |  a6      |response type (fir, paz, fap, etc.)     |

|      |  37-80   | 4     |  a44     |author or source of information         |

|K+2   |    -     | 1     |  f or e  |normalization factor (A0)               |

|K+3   |   1-8    | 1     |  i8      |number of poles                         |

|K+4-N |    -     | 1-4   | 4(f or e)|complex pole and complex error          |

|N+1   |   1-8    | 1     |  i8      |number of zeros                         |

|N+2-M |    -     | 1-4   | 4(f or e)|complex zero and complex error          |

|       Instrument Response Group Using Frequency, Amplitude, Phase (fap)     |

|_____________________________________________________________________________|

|L+2-K |   1-80   | 1     |  a80     | comments (preceded by a "#")           |

|K+1   |   1-12   | 1     |  a12     | response source (theoretical           |

|      |          |       |          | or measured)                           |

|      |  14-15   | 2     |  i2      | sequence number                        |

|      |  17-28   | 2     |  a12     | description (instrument,               |

|      |          |       |          | anti-alias, etc.)                      |

|      |  30-35   | 3     |  a6      | response type (fir, paz, fap, etc.)    |

|      |  37-80   | 4     |  a44     | author or source of information        |

|K+2   |   1-8    | 1     |  i8      | number of fap triplets                 |

|K+3-N |    -     | 1-5   | 5(f or e)| frequency (in deg), amp, phase         |

|      |          |       |          | (in deg), amp err, phase err           |

|       Instrument Response Group Using Finite Impulse Response Filters (fir) |

|_____________________________________________________________________________|

|L+2-K |   1-80   | 1     |  a80     | comments (preceded by a "#")           |

|K+1   |   1-12   | 1     |  a12     | response source (theoretical           |

|      |          |       |          | or measured)                           |

|      |  14-15   | 2     |  i2      | sequence number                        |

|      |  17-28   | 2     |  a12     | description (instrument,               |

|      |          |       |          | anti-alias, etc.)                      |

|      |  30-35   | 3     |  a6      | response type (fir, paz, fap, etc.)    |

|      |  37-80   | 4     |  a44     | author or source of information        |

|K+2   |   1-12   | 1     | f12.4    | input samples/sec                      |

|K+3   |   1-8    | 1     |  i8      | number of numerator coefficients       |

|K+4-N |    -     | 1-2   | 2(f or e)| numerator coefficient and error        |

|N+1   |   1-8    | 1     |  i8      | number of denominator coefficients     |

|N+2-M |    -     | 1-2   | 2(f or e)| denominator coefficient and error      |

|       Additional Response Groups as Needed                                  |

|_____________________________________________________________________________|